Form of Financial Proposal

(On the letter head of the Bidder) Financial Proposal From (Name & Address of the bidder)

Date : <<DD/MM/YYYY>>

To

HR & Admin Manager.

National Highways InVIT Project Managers Private Limited,

Unit-406, D-21 Corporate Park, Sector-21, Dwarka, New Delhi-110077

Sub: Request for Proposal for Selection of Vendor for Supply, Installation, Fixing of Office Infrastructure, Wooden Furniture and Electrical Works at the new Office Premise

Proposal/Rates for supply, installation and fixing of office infrastructure set up and electrical works at new office premises (Address prescribed in RFP) is as under:

| (i) Bill of Quantity (Wooden/Furniture works) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S.no. |  | Description | Unit |  | Quantity |  |  | Rate ₹ |  | Amount ₹ |  |
| 1 |  | Providing & Fixing partition with faming of anodized aluminium (Tabular Section 50 x 50 x 1.291 mm) & two side marine plywood for cupboard/shutter of approved quality (Century Ply - Sainik 710 12 mm) and decorative high pressure laminated sheet (1 mm thick) & wooden moulded beading (2nd class teak wood, 100 x 20 mm) including cutting into required size, jointing with screw adhesive, etc. complete as directed by the Client. |  |  |  |  |  |  |  |  |  |
| (i) |  | 76 mm thick (Partial Partition- 4 feet height-) | Sqft |  | 850 |  |  |  |  |  |  |
| (ii) |  | 76 mm thick (full Partition- Ground to roof) sound proof | Sqft |  | 1160 |  |  |  |  |  |  |
| 2 |  | Wall painting with premium acrylic emulsion paint of interior grade, having VOC (Volatile Organic Compound ) content less than 50 grams/ litre of approved brand and manufacture, including applying additional coats wherever required to achieve even shade & colour and complete as directed by the Client. Two coats | sqm |  | 1000 |  |  |  |  |  |  |
| 3 |  | Providing and supplying executive table is designed with a sleek and functional layout. The table top is constructed with a substantial 36 mm thick MDF board, finished in veneer and polished to a melamine finish, which not only adds durability but also provides an elegant appearance. The gable end and modesty panel, both 18 mm thick, share the same MDF board construction and veneer finish. To enhance its aesthetics and stability, the table features MS (mild steel) chrome-finished legs. This chrome finished legs contribute to both the modem look and the table's sturdiness. The table is equipped with a mobile pedestal storage unit, which includes a drawer + drawer +file drawer configuration, offering a convenient solution for office storage needs. The side unit has a 36mm thick top and an understructure made of 18mm thick MDF board, finished in veneer and polished with a melamine finish. The side unit is divided into two compartments. The features a fixed drawer pedestal with a drawer + drawer + file drawer configuration for organized storage. The 8 mm glass will be provided on the main table completed as directed by the Client. |  |  |  |  |  |  |  |  |  |
| (i) |  | Size :- Main 9.5 (L) x 3.5 (W) x 2.5(H) feet with pedestal, side unit 4.0 (L) x 2.0 (W) x 2.5 (H) feet | each |  | 2 |  |  |  |  |  |  |
| (ii) |  | Size :- Main 8.0 (L) x 3.0 (W) x 2.5(H) feet with pedestal, side unit 4.0 (L) x 2.0 (W) x 2.5 (H) feet | each |  | 5 |  |  |  |  |  |  |
| 4 |  | Providing and placing executive table features a 25mm thick table top made of prelaminated particle board. The edges of the table top are precisely edge banded with a 2mm thick PVC tape, ensuring a finished and durable surface. Both the gable end and the modesty panel of the executive table are constructed using 18mm thick prelaminated particle board, and these components are duly edge banded to create a unified and polished appearance. The side return unit is also made of 18mm thick prelaminated particle board with 2mm thick PVC tape edge banding. This unit is divided into two compartments. The first compartment houses a fixed drawer unit, consisting of one drawer and one filing drawer, providing secure storage for documents and personal items. The second compartment Is designed as an open box with one shelf inside, offering flexible storage options for various office essentials and complete as directed by the Client. |  |  |  |  |  |  |  |  |  |
| (i) |  | Size :- Main 6.0 (L) x 2.5 (W) x 2.5(H) feet with pedestal, side unit 3.0 (L) x 2.0 (W) x 2.5 (H) feet | each |  | 10 |  |  |  |  |  |  |
| 5 |  | Providing and supplying storage unit has an overall size of 900mm (length) x 500mm (width) x 1200mm (height) and is meticulously crafted using PLPB Board (BSL/OSL). The unit is thoughtfully finished with a 2.8mm thick PVC edge banding tape that adheres to the approved laminate shade suitable for interior-grade applications. The unit's top is constructed from a robust 25mm thick material, ensuring durability, while the rest of the body is composed of 18mm thick prelaminated particle board. The storage unit is equipped with three hinged door shutters with 3 shelf inside, providing convenient access to storage compartments. For security and privacy, these shutters are lockable with Godrej or equivalent. The inclusion of 10mm height levellers, which are height-adjustable, ensures the unit's stability even when placed on uneven surfaces. To maintain a polished and finished appearance, all exposed edges are meticulously covered with 2mm thick PVC tape duly edge banding tape and complete as directed by the Client. | Nos. |  | 20 |  |  |  |  |  |  |
| 6 |  | Providing and supplying storage unit is meticulously crafted using PLPB Board (BSL/OSL). The unit is thoughtfully finished with a 2.8mm thick PVC edge banding tape that adheres to the approved laminate shade suitable for interior-grade applications. The unit's top is constructed from a robust 25mm thick material, ensuring durability, while the rest of the body is composed of 18mm thick prelaminated particle board. The storage unit is equipped with hinged door shutters with shelf inside, providing convenient access to storage compartments. For security and privacy, these shutters are lockable Godrej or equivalent. The inclusion of 10mm height levellers, which are height-adjustable, ensures the unit's stability even when placed on uneven surfaces. To maintain a polished and finished appearance, all exposed edges are meticulously covered with 2mm thick PVC tape duly edge banding tape and complete as directed by the Client. | Sqm |  | 100 |  |  |  |  |  |  |
| 7 |  | Providing and supplying U Type of conference table is of an impressive overall size, measuring 14 feet in length, 3 feet mm in width, and 2.5 feet in height. The tabletop is constructed from a substantial 36mm thick PLPB (prelaminated particle board), which not only offers a robust and reliable surface but also adds an elegant finish. The edges of the tabletop are carefully post form for a polished look and to enhance durability. Both the modesty panel and the gable end are made from 18mm thick plpb, duly edge bended with a 2mm thick PVC tape. This design ensures a professional appearance and the sturdiness required for such a substantial table. Each division of table having 1 flapper of size 400mm x 120mm, which allows for electrical provisions. This includes switches, sockets, and data connections, ensuring a well-connected and functional workspace for each division of the table and complete as directed by the Client. | Sqft |  | 110 |  |  |  |  |  |  |
| 8 |  | Dinning-Table The table top should be made of substantial 36 mm thick MDF board with venner finish and complete as directed by the Client. | Sqft |  | 32 |  |  |  |  |  |  |
| 9 |  | Providing & supplying a high-quality 3-seater sofa designed for both comfort and durability. The frame of this sofa is meticulously crafted from well-seasoned teak wood, which has been subjected to a rigorous anti-termite treatment to ensure longevity. The seat is constructed using robust flat springs within the seat frame, which are expertly covered with coir, jute cloth, and markeen, providing a supportive foundation. This seat is further enhanced with a 4-inch thick 1SI marked pinhole rubber cushion, delivering a plush and comfortable sitting experience. The back-end armrests of the sofa me also constructed with precision, utilizing high-quality flat springs and the same coif, jute cloth, nod markeen covering for added strength and support. For added comfort, the beck and armrest are enriched with a 2-inch thick pinhole rubber cushion and high-density PU foam, ensuring that you can relax In style. The entire sofa is elegantly upholstered with letherite tapestry, which not only enhances the visual appeal but also contributor to its overfill durability and complete as directed by the Client. |  |  |  |  |  |  |  |  |  |
| (i) |  | 3 Seater Sofa, Size :- 2005 (L) x 760(D) x 860(H) mm | No. |  | 6 |  |  |  |  |  |  |
| 10 |  | Providing & supplying a high-quality single-seater sofa designed and manufactured to provide utmost comfort and durability. The frame is constructed from high-quality seasoned teak wood, which has been meticulously treated to resist termites, ensuring long-lasting performance. The seat is furnished with premium flat springs within the seat frame, expertly covered with coin, Jute cloth, and markeen, further supported by a 4-inch thick ISI mark pinhole rubber cushion. This combination provides a comfortable and supportive sitting experience. Tne backrest and armrests of the sofa also feature high-quality flat springs, enveloped by coir,jute cloth, and markeen, and are adorned with a 2-inch thick pinhole rubber cushion. To enhance the ergonomic design of the back and armrests, high-density PU foam has been strategically incorporated. This not only offers a plush feel but also maintains the shape and integrity of the sofa over time. To complete the sofa's aesthetics and luxurious feel, it is fully upholstered with letherite tapestry. Letherite, known for its durability and sophisticated appearance, ensures that this single-seater sofa not only feels great to sit on but also adds a touch of elegance to any environment. | No. |  | 4 |  |  |  |  |  |  |
| 11 |  | Providing and supplying a Wooden Center Table with an overall size of 1200 mm in width, 600 mm in depth and 450 mm in height. The frame of the table is crafted from solid teak wood, which is expertly polished in melamine for an elegant and durable finish. The table features an 8mm clear glass top, adding a touch of sophistication to the design. At the bottom of the table, a shelf is provided, winch is constructed Nos. 1.00 using 18 min MDF (Medium-Density Fiberboard) with a 4 mm teak ply layer on lop of bottom shelf. The teak ply is also polished in melamine, allowing you to choose the desired shade to match your interior decor. This Wooden Center Table combines the natural beauty of teak wood with the practicality of a glass top arid a lower shelf. | No. |  | 5 |  |  |  |  |  |  |
| 12 |  | Dinning Table Size :- 1600 (L) x 1000 (W) x 760 (H) mm , table top should be made of substantial 36 mm thick MDF board & 6 nos. Chairs with venner finish and complete as directed by the Client. | Each |  | 2 |  |  |  |  |  |  |
| 13 |  | Door :- Providing and fixing a door (in full Partition Room) an overall size 7 ft high and 3 ft wide, while standard single doors with all Door Hardware & Locks Godrej or equivalent (hinges, locks & handles) and complete as directed by the Client. | Each |  | 5 |  |  |  |  |  |  |
|  |  | Total Amount (A) |  |  |  |  |  |  |  |  |  |
|  |  | GST (B) |  |  |  |  |  |  |  |  |  |
|  |  | Total Amount Incl. GST (i) = (A +B) |  |  |  |  |  |  |  |  |  |
| (ii) Bill of Quantity (electronic works/services) |  |  |  |  |  |  |  |  |  |  |
| Sr. no | Description |  |  | Unit |  | Qty | Rate |  | Amount |  |
| 1 | Wiring for circuit/ submain wiring along with earth wire with the following sizes of FRLS/HFFR PVC insulated copper conductor, single core cable in surface/recessed medium class PVC conduit as required. |  |  |  |  |  |  |  |  |  |
| 1.1 | 2 X 1.5 sq. mm + 1 X 1.5 sq. mm earth wire |  |  | Meter |  | 200 |  |  |  |  |
| 1.2 | 2 X 2.5 sq. mm + 1 X 2.5 sq. mm earth wire |  |  | Meter |  | 200 |  |  |  |  |
| 2 | Supplying and fixing following modular switch/ socket on the existing modular plate & switch box including connections but excluding modular plate etc. as required |  |  |  |  |  |  |  |  |  |
| 2.1 | 5/6 A switch |  |  | Nos |  | 65 |  |  |  |  |
| 2.2 | 3 pin 5/6 A socket outlet |  |  | Nos |  | 55 |  |  |  |  |
| 2.3 | 15/16 A switch |  |  | Nos |  | 16 |  |  |  |  |
| 2.4 | 6 pin 15/16 A socket outlet |  |  | Nos |  | 8 |  |  |  |  |
| 3 | Supplying and fixing following size/ modules, GI box along with modular base & cover plate for modular switches in recess etc. as required. |  |  |  |  |  |  |  |  |  |
| 3.1 | 1 or 2 Module (75mmX75mm) |  |  | Nos |  | 32 |  |  |  |  |
| 3.2 | 8 Module (125mmX125mm) |  |  | Nos |  | 25 |  |  |  |  |
| 4 | Supplying and fixing of following sizes of medium class PVC conduit along with accessories in surface/recess including cutting the wall and making good the same in case of recessed conduit as required |  |  |  |  |  |  |  |  |  |
| 4.1 | 25 mm |  |  | Meter |  | 250 |  |  |  |  |
| 5 | Supplying, Installation, Testing and commissioning of CAT6A Copper Information Outlet (IO) with face plate of color as per site requirement, should have ETL/UL verification program certificate for compliance with ANSI/TIA-568.2-D. All copper Cable and Components should be from same OEM to maintain compatibility and interoperability etc. complete as required. |  |  | Nos |  | 30 |  |  |  |  |
| 6 | Supplying, Installation, Testing and commissioning of RJ45 Connector for CAT 6/6A Cables. RJ45 modular plug supports 4 twisted pairs, 8 positions, 8 connectors. Housing: PC, UL94V-2, transparent color,Use for 24- 26 AWG stranded wires etc. complete as required. |  |  | Nos |  | 60 |  |  |  |  |
| 7 | Supplying, drawing, Testing and commissioning of Cat6A UTP 4 pair, 23 AWG solid copper cable in existing conduit/ on surface, U/FTP, LSZH, NonPlenum, Horizontal (solid) Cable suitable for high speed data networking application supporting upto 10Gbps over a 100 meter channel. The 4 Unshielded Twisted Pairs (UTP) cable with color coded insulation for easy indentification should have FLAME PROPERTIES i.e. Flammability Test - IEC 60332-1,Smoke Density - IEC 61034, LSZH standards compliance: ANSI/TIA-568 C.2, ISO/IEC 11801, IEEE 802.3an, RoHS. Delay Skew should be < 45NS. The outer Cable Diameter should be 7.5 + 2 mm. Cable should have been tested and verified by UL/ ETL etc. complete as required. |  |  |  |  |  |  |  |  |  |
| 7.1 | 2 Run of cable (single run – 450) |  |  | Meter |  | 450 |  |  |  |  |
|  | Total Amount (A) |  |  |  |  |  |  |  |  |  |
|  | GST (B) |  |  |  |  |  |  |  |  |  |
|  | Total Amount Incl. GST (ii ) = (A +B) |  |  |  |  |  |  |  |  |  |

TOTAL COST (iii) = (i+ii)

Total Cost (Rs. In Figures): _____________________________________________________________________

Total Cost (Rs. In Words):______________________________________________________________________

Terms & Conditions:-

- All payments are subject to TDS deduction, if any, applicable as per the law in force.

- In the event of any discrepancy in between figures and words, the amount quoted in words will be considered.

- All the payables shall be subject to deduction of applicable taxes.

- The GST component shall be released on production of proof of payment.

- The material will be branded like Ply, Mica, electrical items, Hardware and Fittings (Century Ply, Hardware -Godrej or Hettich, electrical – Havells or Polycab etc. or as directed by the Client).

Name & Signature of Authorized Signatory with seal